A M Water Services Ltd

Integrated Management System — COSHH Assessment

COSHH Assessment — AdBlue (Diesel Exhaust Fluid)

Document control

| Reference | COSHH_01 |
| --- | --- |
| Revision | 2 |
| Issue date | 1 June 2026 |
| Next review | 1 June 2027 |
| Assessor | Sean Ashton |
| Approved by | Aaron Mason, Director |

Safety Data Sheet: sds/Adblue.pdf (PDF carried alongside this assessment).

Van-pack inclusion: this substance is carried in the field van packs. The Van Packs operative-facing site holds a synchronised copy.

1. Product identification

| Product name | AdBlue (Diesel Exhaust Fluid) |
| --- | --- |
| Supplier | Various automotive suppliers (industry-standard AUS 32 grade to ISO 22241) |
| Active ingredients | Urea solution 32.5% w/w |
| CAS | 57-13-6 (urea) |
| Physical form | Clear liquid |
| Appearance / odour | Colourless to pale yellow, slight ammonia odour |
| Use | Diesel exhaust after-treatment (Selective Catalytic Reduction systems) |

2. Hazard classification

Overall classification:

Not classified as hazardous under CLP

Signal word: Not required — product not classified.

Hazard statements (H-codes)

No hazard classification required under CLP.

Precautionary statements (key P-codes)

No specific precautionary statements applicable.

GHS pictograms

No GHS pictograms apply — product not classified as hazardous.

3. Persons at risk

Vehicle maintenance staff

Plant operators

Refuelling personnel

4. Routes of exposure

Inhalation: Low — not volatile at ambient temperature

Skin contact: Low — may cause mild irritation on prolonged contact

Eye contact: Slight irritation on direct contact

Ingestion: Low — seek medical advice if significant quantity swallowed

5. Workplace exposure limits (HSE EH40/2005)

No UK Workplace Exposure Limits assigned to this product or its constituents.

6. Control measures — hierarchy of controls

Elimination / substitution: evaluated where practicable — see Section 12.

Engineering controls

Proprietary filling equipment

Store in bunded area

Spill kit within 5 m of dispensing point

Administrative controls

Induction training on filling procedures

Equipment inspection monthly

Clean up spills immediately

Personal Protective Equipment

Nitrile gloves

Safety glasses

Coveralls if bulk handling

7. Storage

| Temperature | -11 °C to 30 °C |
| --- | --- |
| Location | Cool, dry area |
| Avoid | Direct sunlight, heat sources, dissimilar metals (e.g. copper, aluminium) |
| Container | Original container |

8. Emergency procedures

First aid

| Route | Action |
| --- | --- |
| Eyes | Rinse with water for several minutes. Seek medical advice if irritation persists. |
| Skin | Wash with soap and water. |
| Inhalation | Move to fresh air (unlikely hazard). |
| Ingestion | Rinse mouth. Drink water. Seek medical advice if unwell. |

Spillage

Contain with absorbent material. Collect in appropriate container. Wash area with water. Dispose via authorised waste contractor.

Emergency contacts

Emergency services: 999

NHS non-emergency: 111

AMWS supervisor: Jason May

HSQE support: Sean Ashton (Onyx Operations)

9. Waste disposal

Non-hazardous waste. Dispose via authorised waste contractor. Standard waste transfer notes apply.

10. Risk rating

| Before controls | With controls |
| --- | --- |
| Low | Minimal |

Risk rating methodology: Severity × Likelihood = Risk (S × L = R). See POL_HSQE_21 — Risk Assessment Policy for the full scoring matrix.

11. Training requirements

Filling procedures (van-mounted dispensing)

Spill response

Storage requirements

12. Monitoring and review

Monthly storage-area inspection

Annual COSHH review

Equipment maintenance checks

13. References

POL_HSQE_08 — COSHH Policy

POL_HSQE_19 — Personal Protective Equipment (PPE) Policy

SOP 8.5 — Hazardous Substances (COSHH)

APP_09 — COSHH Assessments Log

HSE COSHH Regulations 2002 (SI 2002/2677, as amended)

HSE EH40/2005 Workplace Exposure Limits

CLP Regulation (EC) No 1272/2008 (as retained in UK law)

Assessment approved by: Aaron Mason, Director · 1 June 2026

This assessment must be reviewed annually or when circumstances change — whichever is sooner.

Uncontrolled when printed.