A M Water Services Ltd

Integrated Management System — COSHH Assessment

COSHH Assessment — Carplan All Seasons Screenwash

Document control

| Reference | COSHH_03 |
| --- | --- |
| Revision | 2 |
| Issue date | 1 June 2026 |
| Next review | 1 June 2027 |
| Assessor | Sean Ashton |
| Approved by | Aaron Mason, Director |

Safety Data Sheet: sds/carplan-all-seasons-screenwash-5l~GPID_1000199998_COSHH_00.pdf (PDF carried alongside this assessment).

Van-pack inclusion: this substance is carried in the field van packs. The Van Packs operative-facing site holds a synchronised copy.

1. Product identification

| Product name | Carplan All Seasons Screenwash |
| --- | --- |
| Supplier | Tetrosyl Ltd (Carplan brand) |
| Active ingredients | Isopropanol (propan-2-ol) + surfactants + dye |
| CAS | 67-63-0 (isopropanol) |
| Physical form | Liquid |
| Appearance / odour | Coloured liquid (typically blue/yellow) |
| Use | Vehicle windscreen washer fluid — neat or diluted |

2. Hazard classification

Overall classification:

Flammable Liquid Category 3 (depending on concentration)

Signal word: Warning

Hazard statements (H-codes)

| Code | Statement |
| --- | --- |
| H226 | Flammable liquid and vapour. |
| H319 | Causes serious eye irritation. |
| H336 | May cause drowsiness or dizziness. |

Precautionary statements (key P-codes)

| Code | Statement |
| --- | --- |
| P210 | Keep away from heat, hot surfaces, sparks, open flames and other ignition sources. No smoking. |
| P233 | Keep container tightly closed. |
| P280 | Wear protective gloves/protective clothing/eye protection/face protection. |
| P305+351+338 | IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. |
| P403+235 | Store in a well-ventilated place. Keep cool. |

GHS pictograms

GHS02: Flammable

GHS07: Harmful / irritant

3. Persons at risk

Drivers

Vehicle maintenance staff

4. Routes of exposure

Inhalation: Vapour may cause dizziness if concentrated

Skin contact: Defats skin on repeated contact

Eye contact: Causes irritation

Ingestion: Harmful — do not swallow

5. Workplace exposure limits (HSE EH40/2005)

| Substance | 8-hr TWA | 15-min STEL |
| --- | --- | --- |
| Propan-2-ol | 999 mg/m³ (400 ppm) | 1250 mg/m³ (500 ppm) |

6. Control measures — hierarchy of controls

Elimination / substitution: evaluated where practicable — see Section 12.

Engineering controls

Keep container sealed when not in use

Administrative controls

No smoking during filling

Use only intended dispensers

Personal Protective Equipment

Nitrile gloves when handling concentrate

Safety glasses

7. Storage

| Temperature | Cool store |
| --- | --- |
| Location | Fire-resistant store or lockable cabinet |
| Avoid | Heat, sparks, open flames |
| Container | Original sealed container |

8. Emergency procedures

First aid

| Route | Action |
| --- | --- |
| Eyes | Rinse with water for 15 minutes. Seek medical advice. |
| Skin | Wash with soap and water. Remove contaminated clothing. |
| Inhalation | Move to fresh air. Seek medical advice if symptoms persist. |
| Ingestion | Rinse mouth. Do not induce vomiting. Seek medical advice immediately. |

Spillage

Eliminate ignition sources. Absorb with sand/vermiculite. Dispose as hazardous waste.

Fire

Flammable. Use foam, CO₂ or dry powder. Do not use water jet.

Emergency contacts

Emergency services: 999

NHS non-emergency: 111

AMWS supervisor: Jason May

HSQE support: Sean Ashton (Onyx Operations)

9. Waste disposal

Hazardous waste — dispose via authorised waste contractor.

10. Risk rating

| Before controls | With controls |
| --- | --- |
| Moderate | Low |

Risk rating methodology: Severity × Likelihood = Risk (S × L = R). See POL_HSQE_21 — Risk Assessment Policy for the full scoring matrix.

11. Training requirements

Flammable liquids handling

Fire response

12. Monitoring and review

Monthly storage inspection

Annual COSHH review

13. References

POL_HSQE_08 — COSHH Policy

POL_HSQE_19 — Personal Protective Equipment (PPE) Policy

SOP 8.5 — Hazardous Substances (COSHH)

APP_09 — COSHH Assessments Log

HSE COSHH Regulations 2002 (SI 2002/2677, as amended)

HSE EH40/2005 Workplace Exposure Limits

CLP Regulation (EC) No 1272/2008 (as retained in UK law)

Assessment approved by: Aaron Mason, Director · 1 June 2026

This assessment must be reviewed annually or when circumstances change — whichever is sooner.

Uncontrolled when printed.