A M Water Services Ltd

Integrated Management System — COSHH Assessment

COSHH Assessment — Chlorinated Spray & Wipe (Greyland)

Document control

| Reference | COSHH_04 |
| --- | --- |
| Revision | 2 |
| Issue date | 1 June 2026 |
| Next review | 1 June 2027 |
| Assessor | Sean Ashton |
| Approved by | Aaron Mason, Director |

Safety Data Sheet: sds/SDS-62-18-Chlorinated-Spray-Wipe-Greyland-v8.pdf (PDF carried alongside this assessment).

1. Product identification

| Product name | Chlorinated Spray & Wipe (Greyland) |
| --- | --- |
| Supplier | Greyland Ltd |
| Active ingredients | Sodium hypochlorite (< 5%) |
| CAS | 7681-52-9 |
| Physical form | Liquid spray |
| Appearance / odour | Clear/pale yellow liquid, chlorine odour |
| Use | Disinfection of water systems, equipment and surfaces |

2. Hazard classification

Overall classification:

Skin Corrosive Category 1B, Aquatic Acute 1

Signal word: Danger

Hazard statements (H-codes)

| Code | Statement |
| --- | --- |
| H314 | Causes severe skin burns and eye damage. |
| H410 | Very toxic to aquatic life with long lasting effects. |
| EUH031 | Contact with acids liberates toxic gas. |

Precautionary statements (key P-codes)

| Code | Statement |
| --- | --- |
| P260 | Do not breathe dust/fume/gas/mist/vapours/spray. |
| P280 | Wear protective gloves/protective clothing/eye protection/face protection. |
| P301+330+331 | IF SWALLOWED: Rinse mouth. Do NOT induce vomiting. |
| P303+361+353 | IF ON SKIN (or hair): Take off immediately all contaminated clothing. Rinse skin with water. |
| P305+351+338 | IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. |
| P310 | Immediately call a POISON CENTRE or doctor/physician. |
| P405 | Store locked up. |
| P501 | Dispose of contents/container to an authorised waste contractor in accordance with local regulations. |

GHS pictograms

GHS05: Corrosive

GHS09: Environmental hazard

3. Persons at risk

Water-treatment operatives

Maintenance staff

Cleaning personnel

4. Routes of exposure

Inhalation: Vapour causes respiratory irritation

Skin contact: Severe burns

Eye contact: Severe damage — risk of blindness

Ingestion: Severe corrosive effects on digestive tract

5. Workplace exposure limits (HSE EH40/2005)

| Substance | 8-hr TWA | 15-min STEL |
| --- | --- | --- |
| Chlorine (released) | 0.5 ppm | 1 ppm |

6. Control measures — hierarchy of controls

Elimination / substitution: evaluated where practicable — see Section 12.

Engineering controls

Use in well-ventilated areas

Eyewash station and safety shower within 10 m

Store in bunded area

Administrative controls

Never mix with acids or other chemicals

Use minimum quantity required

Restricted-access storage

Labelled spray bottles only

Personal Protective Equipment

PVC or nitrile chemical-resistant gloves

Chemical safety goggles

Face shield when decanting

Chemical-resistant apron

7. Storage

| Temperature | 5–25 °C |
| --- | --- |
| Location | Cool, ventilated area away from sunlight |
| Avoid | Acids, ammonia, metals, organics, heat |
| Container | Original sealed container — do not decant into unlabelled bottles |

8. Emergency procedures

First aid

| Route | Action |
| --- | --- |
| Eyes | Rinse with water for at least 15 minutes, holding eyelids apart. Seek immediate medical attention. |
| Skin | Remove contaminated clothing. Wash with plenty of water for 15 minutes. Seek medical attention. |
| Inhalation | Move to fresh air. Seek medical attention if symptoms persist. |
| Ingestion | DO NOT induce vomiting. Rinse mouth. Give small sips of water if conscious. Seek immediate medical attention. |

Spillage

Evacuate area. PPE as above. Dilute with water. Absorb with inert material. Neutralise with sodium thiosulphate if available. Dispose as hazardous waste.

Fire

Not flammable but releases toxic fumes (Cl₂) in fire. Use water spray to cool containers.

Emergency contacts

Emergency services: 999

NHS non-emergency: 111

AMWS supervisor: Jason May

HSQE support: Sean Ashton (Onyx Operations)

9. Waste disposal

Hazardous waste. Dispose via authorised chemical-waste contractor. EWC code 20 01 29* (detergents containing hazardous substances).

10. Risk rating

| Before controls | With controls |
| --- | --- |
| High | Low |

Risk rating methodology: Severity × Likelihood = Risk (S × L = R). See POL_HSQE_21 — Risk Assessment Policy for the full scoring matrix.

11. Training requirements

COSHH briefing

Spill and emergency procedures

Decanting safely

Fire awareness

12. Monitoring and review

Weekly visual check of storage area

Bi-monthly PPE condition check

Annual COSHH review

13. References

POL_HSQE_08 — COSHH Policy

POL_HSQE_19 — Personal Protective Equipment (PPE) Policy

SOP 8.5 — Hazardous Substances (COSHH)

APP_09 — COSHH Assessments Log

HSE COSHH Regulations 2002 (SI 2002/2677, as amended)

HSE EH40/2005 Workplace Exposure Limits

CLP Regulation (EC) No 1272/2008 (as retained in UK law)

Assessment approved by: Aaron Mason, Director · 1 June 2026

This assessment must be reviewed annually or when circumstances change — whichever is sooner.

Uncontrolled when printed.