A M Water Services Ltd

Integrated Management System — COSHH Assessment

COSHH Assessment — Comma Eurolite 10W-40 Semi-Synthetic Motor Oil

Document control

| Reference | COSHH_05 |
| --- | --- |
| Revision | 2 |
| Issue date | 1 June 2026 |
| Next review | 1 June 2027 |
| Assessor | Sean Ashton |
| Approved by | Aaron Mason, Director |

Safety Data Sheet: sds/EUL-L.pdf (PDF carried alongside this assessment).

Van-pack inclusion: this substance is carried in the field van packs. The Van Packs operative-facing site holds a synchronised copy.

1. Product identification

| Product name | Comma Eurolite 10W-40 Semi-Synthetic Motor Oil |
| --- | --- |
| Supplier | Comma Oil & Chemicals Ltd |
| Active ingredients | Highly-refined mineral base oil + performance additives |
| CAS | 64742-54-7 (base oil) |
| Physical form | Viscous liquid |
| Appearance / odour | Amber/brown liquid |
| Use | Engine lubricant for petrol and diesel vehicles |

2. Hazard classification

Overall classification:

Not classified as hazardous (fresh oil). Used oil may contain hazardous combustion products.

Signal word: Not required — product not classified.

Hazard statements (H-codes)

No hazard classification required under CLP.

Precautionary statements (key P-codes)

| Code | Statement |
| --- | --- |
| P264 | Wash thoroughly after handling. |
| P280 | Wear protective gloves/protective clothing/eye protection/face protection. |
| P501 | Dispose of contents/container to an authorised waste contractor in accordance with local regulations. |

GHS pictograms

No GHS pictograms apply — product not classified as hazardous.

3. Persons at risk

Vehicle maintenance staff

Plant operators

4. Routes of exposure

Inhalation: Low — mist possible during pouring

Skin contact: Repeated/prolonged contact may cause dermatitis

Eye contact: Mild irritation

Ingestion: Low

5. Workplace exposure limits (HSE EH40/2005)

| Substance | 8-hr TWA | 15-min STEL |
| --- | --- | --- |
| Oil mist, mineral | 5 mg/m³ | 10 mg/m³ |

6. Control measures — hierarchy of controls

Elimination / substitution: evaluated where practicable — see Section 12.

Engineering controls

Drip trays under dispensing points

Bunded storage

Administrative controls

Avoid skin contact — used oil is a recognised carcinogen (see HSE INDG35)

Change contaminated clothing promptly

Personal Protective Equipment

Nitrile gloves

Safety glasses

Overalls

7. Storage

| Temperature | Ambient |
| --- | --- |
| Location | Dry store, bunded if bulk |
| Avoid | Strong oxidisers |
| Container | Original container |

8. Emergency procedures

First aid

| Route | Action |
| --- | --- |
| Eyes | Rinse with water for 15 minutes. Seek medical advice if irritation persists. |
| Skin | Wash with soap and water. Apply emollient. Seek medical advice if dermatitis develops. |
| Inhalation | Move to fresh air. |
| Ingestion | Do not induce vomiting. Seek medical advice. |

Spillage

Absorb with sand or proprietary oil absorbent. Dispose as special/hazardous waste.

Fire

Combustible if heated. Use foam, dry powder or CO₂.

Emergency contacts

Emergency services: 999

NHS non-emergency: 111

AMWS supervisor: Jason May

HSQE support: Sean Ashton (Onyx Operations)

9. Waste disposal

Used engine oil is hazardous (EWC 13 02). Contain and send via authorised waste oil collector.

10. Risk rating

| Before controls | With controls |
| --- | --- |
| Low | Minimal |

Risk rating methodology: Severity × Likelihood = Risk (S × L = R). See POL_HSQE_21 — Risk Assessment Policy for the full scoring matrix.

11. Training requirements

Safe oil handling

Used-oil segregation

Skin-care awareness (used oil carcinogen)

12. Monitoring and review

Monthly storage-area inspection

Annual COSHH review

13. References

POL_HSQE_08 — COSHH Policy

POL_HSQE_19 — Personal Protective Equipment (PPE) Policy

SOP 8.5 — Hazardous Substances (COSHH)

APP_09 — COSHH Assessments Log

HSE COSHH Regulations 2002 (SI 2002/2677, as amended)

HSE EH40/2005 Workplace Exposure Limits

CLP Regulation (EC) No 1272/2008 (as retained in UK law)

Assessment approved by: Aaron Mason, Director · 1 June 2026

This assessment must be reviewed annually or when circumstances change — whichever is sooner.

Uncontrolled when printed.