A M Water Services Ltd

Integrated Management System — COSHH Assessment

COSHH Assessment — Hy-Ram Chlorine Disinfection Tablets

Document control

| Reference | COSHH_07 |
| --- | --- |
| Revision | 2 |
| Issue date | 1 June 2026 |
| Next review | 1 June 2027 |
| Assessor | Sean Ashton |
| Approved by | Aaron Mason, Director |

Safety Data Sheet: sds/1084761.pdf (PDF carried alongside this assessment).

Van-pack inclusion: this substance is carried in the field van packs. The Van Packs operative-facing site holds a synchronised copy.

1. Product identification

| Product name | Hy-Ram Chlorine Disinfection Tablets |
| --- | --- |
| Supplier | Hy-Ram Engineering Ltd |
| Active ingredients | Sodium dichloroisocyanurate (NaDCC) |
| CAS | 2893-78-9 |
| Physical form | Effervescent tablets |
| Appearance / odour | White tablets, chlorine odour when dissolved |
| Use | Disinfection of potable water mains following repair/installation |

2. Hazard classification

Overall classification:

Acute Toxicity (oral) Cat 4, Eye Irritation Cat 2, STOT SE 3 (respiratory), Aquatic Acute 1

Signal word: Warning

Hazard statements (H-codes)

| Code | Statement |
| --- | --- |
| H302 | Harmful if swallowed. |
| H319 | Causes serious eye irritation. |
| H335 | May cause respiratory irritation. |
| H410 | Very toxic to aquatic life with long lasting effects. |
| EUH031 | Contact with acids liberates toxic gas. |

Precautionary statements (key P-codes)

| Code | Statement |
| --- | --- |
| P261 | Avoid breathing dust/fume/gas/mist/vapours/spray. |
| P264 | Wash thoroughly after handling. |
| P271 | Use only outdoors or in a well-ventilated area. |
| P273 | Avoid release to the environment. |
| P280 | Wear protective gloves/protective clothing/eye protection/face protection. |
| P301+312 | IF SWALLOWED: Call a POISON CENTRE or doctor/physician if you feel unwell. |
| P305+351+338 | IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. |
| P501 | Dispose of contents/container to an authorised waste contractor in accordance with local regulations. |

GHS pictograms

GHS07: Harmful / irritant

GHS09: Environmental hazard

3. Persons at risk

Water-main operatives

Mains-commissioning personnel

4. Routes of exposure

Inhalation: Dust/fume may irritate respiratory tract

Skin contact: Irritation on prolonged contact

Eye contact: Causes irritation

Ingestion: Harmful if swallowed

5. Workplace exposure limits (HSE EH40/2005)

| Substance | 8-hr TWA | 15-min STEL |
| --- | --- | --- |
| Chlorine (released) | 0.5 ppm | 1 ppm |

6. Control measures — hierarchy of controls

Elimination / substitution: evaluated where practicable — see Section 12.

Engineering controls

Dissolve only in approved chlorination rig

Never add water to tablet — add tablet to water

Administrative controls

Use only for purpose specified

Keep away from acids

One tablet per intended volume only

Personal Protective Equipment

Nitrile gloves

Chemical goggles

Dust mask when handling dry tablets

7. Storage

| Temperature | Ambient, dry |
| --- | --- |
| Location | Original container only. Locked cabinet. |
| Avoid | Moisture, acids, ammonia, combustibles, direct sunlight |
| Container | Original sealed drum — do not decant |

8. Emergency procedures

First aid

| Route | Action |
| --- | --- |
| Eyes | Rinse with water for 15 minutes. Seek medical advice. |
| Skin | Wash with soap and water. |
| Inhalation | Move to fresh air. Seek medical advice if symptoms persist. |
| Ingestion | Rinse mouth. Drink water. Do NOT induce vomiting. Seek medical attention. |

Spillage

Dry spill: sweep carefully (avoid dust) into container for disposal. Wet spill: dilute with large volumes of water. Do not return to container.

Fire

Not combustible but supports combustion. Use water spray. DO NOT use dry powder containing ammonium compounds.

Emergency contacts

Emergency services: 999

NHS non-emergency: 111

AMWS supervisor: Jason May

HSQE support: Sean Ashton (Onyx Operations)

9. Waste disposal

Hazardous waste. Dispose via authorised chemical-waste contractor.

10. Risk rating

| Before controls | With controls |
| --- | --- |
| High | Low |

Risk rating methodology: Severity × Likelihood = Risk (S × L = R). See POL_HSQE_21 — Risk Assessment Policy for the full scoring matrix.

11. Training requirements

Water-main chlorination procedure

COSHH briefing

Spill response

12. Monitoring and review

Weekly visual stock check

Expiry-date check monthly

Annual COSHH review

13. References

POL_HSQE_08 — COSHH Policy

POL_HSQE_19 — Personal Protective Equipment (PPE) Policy

SOP 8.5 — Hazardous Substances (COSHH)

APP_09 — COSHH Assessments Log

HSE COSHH Regulations 2002 (SI 2002/2677, as amended)

HSE EH40/2005 Workplace Exposure Limits

CLP Regulation (EC) No 1272/2008 (as retained in UK law)

Assessment approved by: Aaron Mason, Director · 1 June 2026

This assessment must be reviewed annually or when circumstances change — whichever is sooner.

Uncontrolled when printed.