A M Water Services Ltd

Integrated Management System — COSHH Assessment

COSHH Assessment — Line-Marking Paint — Red Aerosol

Document control

| Reference | COSHH_09 |
| --- | --- |
| Revision | 2 |
| Issue date | 1 June 2026 |
| Next review | 1 June 2027 |
| Assessor | Sean Ashton |
| Approved by | Aaron Mason, Director |

Safety Data Sheet: sds/LMRD_5060253470703.pdf (PDF carried alongside this assessment).

Van-pack inclusion: this substance is carried in the field van packs. The Van Packs operative-facing site holds a synchronised copy.

1. Product identification

| Product name | Line-Marking Paint — Red Aerosol |
| --- | --- |
| Supplier | Various (line-marking product) |
| Active ingredients | Hydrocarbon solvents + pigment + propellant |
| CAS | Mixture — see SDS |
| Physical form | Aerosol |
| Appearance / odour | Red spray |
| Use | Temporary red line-marking for site demarcation, danger zones, services |

2. Hazard classification

Overall classification:

Aerosol Cat 1, Aspiration Cat 1 (solvent base)

Signal word: Danger

Hazard statements (H-codes)

| Code | Statement |
| --- | --- |
| H222 | Extremely flammable aerosol. |
| H229 | Pressurised container: may burst if heated. |
| H304 | May be fatal if swallowed and enters airways. |
| H315 | Causes skin irritation. |
| H319 | Causes serious eye irritation. |
| H336 | May cause drowsiness or dizziness. |
| H411 | Toxic to aquatic life with long lasting effects. |

Precautionary statements (key P-codes)

| Code | Statement |
| --- | --- |
| P210 | Keep away from heat, hot surfaces, sparks, open flames and other ignition sources. No smoking. |
| P211 | Do not spray on an open flame or other ignition source. |
| P251 | Do not pierce or burn, even after use. |
| P261 | Avoid breathing dust/fume/gas/mist/vapours/spray. |
| P280 | Wear protective gloves/protective clothing/eye protection/face protection. |
| P410+412 | Protect from sunlight. Do not expose to temperatures exceeding 50 °C. |
| P501 | Dispose of contents/container to an authorised waste contractor in accordance with local regulations. |

GHS pictograms

GHS02: Flammable

GHS07: Harmful / irritant

GHS08: Health hazard (serious)

GHS09: Environmental hazard

3. Persons at risk

Line-marking operatives

Site personnel

4. Routes of exposure

Inhalation: Vapour/mist — may cause dizziness

Skin contact: Mild irritation

Eye contact: Irritation

Ingestion: Aspiration hazard

5. Workplace exposure limits (HSE EH40/2005)

No UK Workplace Exposure Limits assigned to this product or its constituents.

6. Control measures — hierarchy of controls

Elimination / substitution: evaluated where practicable — see Section 12.

Engineering controls

Use in open air wherever possible

Administrative controls

No smoking

No hot work nearby

Personal Protective Equipment

Nitrile gloves

Safety glasses

7. Storage

| Temperature | Below 50 °C |
| --- | --- |
| Location | Cool ventilated store |
| Avoid | Heat, sparks, direct sunlight |
| Container | Original aerosol can |

8. Emergency procedures

First aid

| Route | Action |
| --- | --- |
| Eyes | Rinse with water for 15 minutes. Seek medical advice. |
| Skin | Wash with soap and water. |
| Inhalation | Move to fresh air. |
| Ingestion | Do not induce vomiting. Seek immediate medical attention (aspiration risk). |

Spillage

Eliminate ignition sources. Absorb with inert material. Dispose as hazardous waste.

Fire

Flammable aerosol. Use foam, CO₂ or dry powder.

Emergency contacts

Emergency services: 999

NHS non-emergency: 111

AMWS supervisor: Jason May

HSQE support: Sean Ashton (Onyx Operations)

9. Waste disposal

Empty aerosols are hazardous (EWC 16 05 04*).

10. Risk rating

| Before controls | With controls |
| --- | --- |
| Moderate | Low |

Risk rating methodology: Severity × Likelihood = Risk (S × L = R). See POL_HSQE_21 — Risk Assessment Policy for the full scoring matrix.

11. Training requirements

Aerosol handling

12. Monitoring and review

Weekly van-pack inspection

Annual COSHH review

13. References

POL_HSQE_08 — COSHH Policy

POL_HSQE_19 — Personal Protective Equipment (PPE) Policy

SOP 8.5 — Hazardous Substances (COSHH)

APP_09 — COSHH Assessments Log

HSE COSHH Regulations 2002 (SI 2002/2677, as amended)

HSE EH40/2005 Workplace Exposure Limits

CLP Regulation (EC) No 1272/2008 (as retained in UK law)

Assessment approved by: Aaron Mason, Director · 1 June 2026

This assessment must be reviewed annually or when circumstances change — whichever is sooner.

Uncontrolled when printed.