A M Water Services Ltd

Integrated Management System — COSHH Assessment

COSHH Assessment — Traffic Film Remover (Orbit — Würth)

Document control

| Reference | COSHH_10 |
| --- | --- |
| Revision | 2 |
| Issue date | 1 June 2026 |
| Next review | 1 June 2027 |
| Assessor | Sean Ashton |
| Approved by | Aaron Mason, Director |

Safety Data Sheet: sds/C312.pdf (PDF carried alongside this assessment).

Van-pack inclusion: this substance is carried in the field van packs. The Van Packs operative-facing site holds a synchronised copy.

1. Product identification

| Product name | Traffic Film Remover (Orbit — Würth) |
| --- | --- |
| Supplier | Würth UK Ltd |
| Active ingredients | Alkaline cleaner — contains sodium hydroxide + anionic surfactants |
| CAS | 1310-73-2 (NaOH) |
| Physical form | Liquid concentrate |
| Appearance / odour | Clear/pale-coloured liquid |
| Use | Exterior vehicle pre-wash for removal of road film |

2. Hazard classification

Overall classification:

Skin Corrosive Cat 1B, Eye Damage Cat 1

Signal word: Danger

Hazard statements (H-codes)

| Code | Statement |
| --- | --- |
| H314 | Causes severe skin burns and eye damage. |

Precautionary statements (key P-codes)

| Code | Statement |
| --- | --- |
| P260 | Do not breathe dust/fume/gas/mist/vapours/spray. |
| P264 | Wash thoroughly after handling. |
| P280 | Wear protective gloves/protective clothing/eye protection/face protection. |
| P301+330+331 | IF SWALLOWED: Rinse mouth. Do NOT induce vomiting. |
| P303+361+353 | IF ON SKIN (or hair): Take off immediately all contaminated clothing. Rinse skin with water. |
| P305+351+338 | IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. |
| P310 | Immediately call a POISON CENTRE or doctor/physician. |
| P405 | Store locked up. |
| P501 | Dispose of contents/container to an authorised waste contractor in accordance with local regulations. |

GHS pictograms

GHS05: Corrosive

3. Persons at risk

Vehicle-cleaning operatives

4. Routes of exposure

Inhalation: Mist irritates respiratory tract

Skin contact: Severe burns

Eye contact: Severe damage — irreversible possible

Ingestion: Severe corrosive effects

5. Workplace exposure limits (HSE EH40/2005)

| Substance | 8-hr TWA | 15-min STEL |
| --- | --- | --- |
| Sodium hydroxide (mist) | 2 mg/m³ (15-min STEL) | — |

6. Control measures — hierarchy of controls

Elimination / substitution: evaluated where practicable — see Section 12.

Engineering controls

Use in designated wash bay with bund

Eyewash station within 10 m

Pressure washer with fan nozzle to reduce splash-back

Administrative controls

Always dilute per instructions — never use neat

Never mix with acids or other chemicals

Personal Protective Equipment

Nitrile or PVC chemical gloves

Chemical goggles or face shield

Chemical-resistant apron

Wellingtons

7. Storage

| Temperature | 5–25 °C |
| --- | --- |
| Location | Bunded area, away from acids |
| Avoid | Acids, aluminium, zinc, organics |
| Container | Original sealed container |

8. Emergency procedures

First aid

| Route | Action |
| --- | --- |
| Eyes | Rinse with water for at least 15 minutes. Remove contact lenses. Seek immediate medical attention. |
| Skin | Remove contaminated clothing. Wash with plenty of water for 15 minutes. Seek medical attention. |
| Inhalation | Move to fresh air. Seek medical advice if symptoms persist. |
| Ingestion | DO NOT induce vomiting. Rinse mouth. Give water if conscious. Seek immediate medical attention. |

Spillage

Dilute with water. Neutralise with dilute acid (e.g. citric acid). Absorb with inert material. Dispose as hazardous waste.

Fire

Not flammable.

Emergency contacts

Emergency services: 999

NHS non-emergency: 111

AMWS supervisor: Jason May

HSQE support: Sean Ashton (Onyx Operations)

9. Waste disposal

Hazardous waste (alkaline). Dispose via authorised contractor.

10. Risk rating

| Before controls | With controls |
| --- | --- |
| High | Low |

Risk rating methodology: Severity × Likelihood = Risk (S × L = R). See POL_HSQE_21 — Risk Assessment Policy for the full scoring matrix.

11. Training requirements

Corrosive chemical handling

Spill response

PPE fit

12. Monitoring and review

Weekly PPE condition check

Annual COSHH review

13. References

POL_HSQE_08 — COSHH Policy

POL_HSQE_19 — Personal Protective Equipment (PPE) Policy

SOP 8.5 — Hazardous Substances (COSHH)

APP_09 — COSHH Assessments Log

HSE COSHH Regulations 2002 (SI 2002/2677, as amended)

HSE EH40/2005 Workplace Exposure Limits

CLP Regulation (EC) No 1272/2008 (as retained in UK law)

Assessment approved by: Aaron Mason, Director · 1 June 2026

This assessment must be reviewed annually or when circumstances change — whichever is sooner.

Uncontrolled when printed.