A M Water Services Ltd

Integrated Management System — COSHH Assessment

COSHH Assessment — BP Unleaded Petrol (E10 — 95 RON)

Document control

| Reference | COSHH_23 |
| --- | --- |
| Revision | 2 |
| Issue date | 1 June 2026 |
| Next review | 1 June 2027 |
| Assessor | Sean Ashton |
| Approved by | Aaron Mason, Director |

Safety Data Sheet: https://msdspds.bp.com/ (supplier website).

Van-pack inclusion: this substance is carried in the field van packs. The Van Packs operative-facing site holds a synchronised copy.

1. Product identification

| Product name | BP Unleaded Petrol (E10 — 95 RON) |
| --- | --- |
| Supplier | BP Oil UK Ltd (bulk) / Forecourt refuel |
| Active ingredients | Hydrocarbons (C4–C12) + ethanol (≤10%) + additives. Contains **benzene** (< 1% v/v) |
| CAS | 86290-81-5 (gasoline) · 71-43-2 (benzene) · 64-17-5 (ethanol) |
| Physical form | Liquid |
| Appearance / odour | Clear liquid, characteristic petroleum odour |
| Use | Fuel for petrol-powered tools (Stihl disc-cutters, breakers, water pumps) |

2. Hazard classification

Overall classification:

**Extremely flammable**. Carcinogen Cat 1A (contains benzene). Germ Cell Mutagenicity Cat 1B. Aspiration Cat 1. STOT SE 3. Reproductive Toxicity Cat 2. Aquatic Chronic 2.

Signal word: Danger

Hazard statements (H-codes)

| Code | Statement |
| --- | --- |
| H224 | Extremely flammable liquid and vapour. |
| H304 | May be fatal if swallowed and enters airways. |
| H315 | Causes skin irritation. |
| H336 | May cause drowsiness or dizziness. |
| H340 | May cause genetic defects. |
| H350 | May cause cancer. |
| H361 | Suspected of damaging fertility or the unborn child. |
| H411 | Toxic to aquatic life with long lasting effects. |

Precautionary statements (key P-codes)

| Code | Statement |
| --- | --- |
| P201 | Obtain special instructions before use. |
| P210 | Keep away from heat, hot surfaces, sparks, open flames and other ignition sources. No smoking. |
| P240 | Ground and bond container and receiving equipment. |
| P243 | Take precautionary measures against static discharge. |
| P280 | Wear protective gloves/protective clothing/eye protection/face protection. |
| P301+310 | IF SWALLOWED: Immediately call a POISON CENTRE or doctor/physician. |
| P331 | Do NOT induce vomiting. |
| P370+378 | In case of fire: Use foam, dry powder or CO₂ to extinguish. Do not use water jet. |
| P403+235 | Store in a well-ventilated place. Keep cool. |
| P405 | Store locked up. |
| P501 | Dispose of contents/container to an authorised waste contractor in accordance with local regulations. |

GHS pictograms

GHS02: Flammable

GHS07: Harmful / irritant

GHS08: Health hazard (serious)

GHS09: Environmental hazard

3. Persons at risk

Operatives using petrol-powered tools

Refuelling personnel

Anyone in vicinity of open containers

4. Routes of exposure

Inhalation: Vapour may cause dizziness, drowsiness, headache. **Benzene is a known human carcinogen** — any exposure is to be minimised.

Skin contact: Defats skin — repeated contact causes dermatitis. Benzene can be absorbed through skin.

Eye contact: Causes irritation

Ingestion: **May be fatal** if aspirated into lungs. Systemically toxic.

5. Workplace exposure limits (HSE EH40/2005)

| Substance | 8-hr TWA | 15-min STEL |
| --- | --- | --- |
| Petrol vapour (hydrocarbon) | 900 mg/m³ (varies — see EH40) | — |
| Benzene | 1 ppm (3.25 mg/m³) | — |
| Ethanol | 1000 ppm (1920 mg/m³) | — |

6. Control measures — hierarchy of controls

Elimination / substitution: evaluated where practicable — see Section 12.

Engineering controls

**Refuel outdoors or in open air only** — never indoors or in confined space

Use bonded/earthed metal cans (or UN-approved plastic containers) — never glass or non-approved vessels

Keep container upright, tightly closed when not dispensing

Funnel or dispensing nozzle to minimise splashing

No open flames, hot surfaces or ignition sources within 3 m during refuelling

Administrative controls

Refuel tools only when cold (allow 5 min cool-down after operation)

Carry max 20 L per van (regulatory limit) in approved containers

Never siphon petrol by mouth

No smoking, no mobile-phone use during refuelling

Engine off during refuelling

Immediate wash-off of any skin contamination

Personal Protective Equipment

Nitrile gloves (replace if saturated)

Safety glasses

Antistatic footwear advisable

7. Storage

| Temperature | Below 30 °C (ideally below 20 °C) |
| --- | --- |
| Location | Dedicated ventilated flammable-storage cabinet or external metal fuel store — segregated from the cab |
| Avoid | Ignition sources, heat, direct sunlight, oxidisers, naked flames, static electricity |
| Container | UN-approved 5–20 L metal can or HSE-approved plastic container only |

8. Emergency procedures

First aid

| Route | Action |
| --- | --- |
| Eyes | Rinse with water for 15 minutes. Seek medical advice. |
| Skin | Remove contaminated clothing. Wash with soap and water. Seek medical advice if irritation persists. |
| Inhalation | Move to fresh air immediately. If breathing difficulty or unconsciousness, call 999. |
| Ingestion | **DO NOT induce vomiting** (aspiration risk). Rinse mouth. Seek immediate medical attention — call 999. |

Spillage

**Eliminate all ignition sources immediately.** Evacuate non-essential personnel. Ventilate area. Absorb with sand or vermiculite (do not use combustible absorbents). Place in sealed metal container labelled 'Petrol-contaminated waste'. Do not release to drains.

Fire

**Extremely flammable liquid.** Use foam, dry powder, CO₂ or water spray (fine). **Do not use water jet.** Evacuate and call 999.

Emergency contacts

Emergency services: 999

NHS non-emergency: 111

AMWS supervisor: Jason May

HSQE support: Sean Ashton (Onyx Operations)

9. Waste disposal

Hazardous waste (EWC 13 07 02*). Dispose via authorised waste-fuel contractor. Contaminated absorbent is also hazardous.

10. Risk rating

| Before controls | With controls |
| --- | --- |
| High | Moderate |

Risk rating methodology: Severity × Likelihood = Risk (S × L = R). See POL_HSQE_21 — Risk Assessment Policy for the full scoring matrix.

11. Training requirements

Safe refuelling procedure

Flammable-liquids awareness (DSEAR)

Fire extinguisher use

Spill response — benzene-carrying fuels

First-aid response for inhalation / aspiration

12. Monitoring and review

Weekly van-pack visual inspection — containers and labelling

Monthly check of storage-area ventilation and fire-extinguisher status

Annual COSHH review

Ongoing health surveillance for operatives with frequent exposure (per APP_08)

13. References

POL_HSQE_08 — COSHH Policy

POL_HSQE_19 — Personal Protective Equipment (PPE) Policy

SOP 8.5 — Hazardous Substances (COSHH)

APP_09 — COSHH Assessments Log

HSE COSHH Regulations 2002 (SI 2002/2677, as amended)

HSE EH40/2005 Workplace Exposure Limits

CLP Regulation (EC) No 1272/2008 (as retained in UK law)

Assessment approved by: Aaron Mason, Director · 1 June 2026

This assessment must be reviewed annually or when circumstances change — whichever is sooner.

Uncontrolled when printed.