A M Water Services Ltd

Integrated Management System — COSHH Assessment

COSHH Assessment — BP Diesel Fuel (EN590 / B7)

Document control

| Reference | COSHH_24 |
| --- | --- |
| Revision | 2 |
| Issue date | 1 June 2026 |
| Next review | 1 June 2027 |
| Assessor | Sean Ashton |
| Approved by | Aaron Mason, Director |

Safety Data Sheet: https://msdspds.bp.com/ (supplier website).

Van-pack inclusion: this substance is carried in the field van packs. The Van Packs operative-facing site holds a synchronised copy.

1. Product identification

| Product name | BP Diesel Fuel (EN590 / B7) |
| --- | --- |
| Supplier | BP Oil UK Ltd / Forecourt refuel |
| Active ingredients | Middle-distillate hydrocarbons (C9–C20) + fatty-acid methyl esters (FAME, up to 7%) + additives |
| CAS | 68334-30-5 (gas oil) · 68476-30-2 (diesel fuel No. 2) |
| Physical form | Liquid |
| Appearance / odour | Yellow-to-amber liquid, characteristic fuel odour |
| Use | Fuel for van, grab lorry, and diesel-powered site plant |

2. Hazard classification

Overall classification:

Flammable Liquid Cat 3. Acute Toxicity (inhalation) Cat 4. Skin Irritation Cat 2. Carcinogen Cat 2. Aspiration Cat 1. STOT RE 2. Aquatic Chronic 2.

Signal word: Danger

Hazard statements (H-codes)

| Code | Statement |
| --- | --- |
| H226 | Flammable liquid and vapour. |
| H304 | May be fatal if swallowed and enters airways. |
| H315 | Causes skin irritation. |
| H332 | Harmful if inhaled. |
| H351 | Suspected of causing cancer. |
| H373 | May cause damage to organs through prolonged or repeated exposure. |
| H411 | Toxic to aquatic life with long lasting effects. |

Precautionary statements (key P-codes)

| Code | Statement |
| --- | --- |
| P210 | Keep away from heat, hot surfaces, sparks, open flames and other ignition sources. No smoking. |
| P273 | Avoid release to the environment. |
| P280 | Wear protective gloves/protective clothing/eye protection/face protection. |
| P301+310 | IF SWALLOWED: Immediately call a POISON CENTRE or doctor/physician. |
| P303+361+353 | IF ON SKIN (or hair): Take off immediately all contaminated clothing. Rinse skin with water. |
| P331 | Do NOT induce vomiting. |
| P403+235 | Store in a well-ventilated place. Keep cool. |
| P405 | Store locked up. |
| P501 | Dispose of contents/container to an authorised waste contractor in accordance with local regulations. |

GHS pictograms

GHS02: Flammable

GHS07: Harmful / irritant

GHS08: Health hazard (serious)

GHS09: Environmental hazard

3. Persons at risk

Drivers

Refuelling personnel

Plant operators

4. Routes of exposure

Inhalation: Vapour/mist may cause respiratory irritation and drowsiness. Harmful if inhaled.

Skin contact: Causes irritation. Prolonged/repeated contact is a suspected carcinogen.

Eye contact: Causes irritation

Ingestion: **May be fatal** if aspirated into lungs.

5. Workplace exposure limits (HSE EH40/2005)

| Substance | 8-hr TWA | 15-min STEL |
| --- | --- | --- |
| Diesel-engine exhaust emissions (DEEE) | Carcinogenic — minimise exposure | — |
| Fuel mist (oil mist) | 5 mg/m³ | 10 mg/m³ |

6. Control measures — hierarchy of controls

Elimination / substitution: evaluated where practicable — see Section 12.

Engineering controls

Refuel outdoors where possible

Use appropriate dispensing equipment (pumps, funnels with strainers)

Drip tray and spill kit at all dispensing points

Adequate ventilation during refuelling

Administrative controls

Refuel with engine off

No smoking / open flames / hot work during refuelling

Immediate wash-off of skin contamination

Avoid prolonged/repeated skin contact

Contaminated clothing removed and laundered separately

Personal Protective Equipment

Nitrile gloves

Safety glasses

Overalls (change if saturated)

7. Storage

| Temperature | Below 30 °C |
| --- | --- |
| Location | Bunded fuel store or approved on-vehicle tank. Segregated from petrol and oxidisers. |
| Avoid | Heat, sparks, ignition sources, oxidisers |
| Container | UN-approved metal or plastic diesel container / bulk tank |

8. Emergency procedures

First aid

| Route | Action |
| --- | --- |
| Eyes | Rinse with water for 15 minutes. Seek medical advice. |
| Skin | Remove contaminated clothing. Wash with soap and water. |
| Inhalation | Move to fresh air. Seek medical advice if symptoms persist. |
| Ingestion | **DO NOT induce vomiting** (aspiration risk). Rinse mouth. Seek immediate medical attention. |

Spillage

Eliminate ignition sources. Absorb with sand or proprietary oil-absorbent. Place in sealed container labelled 'Diesel-contaminated waste'. Prevent runoff to drains/watercourses.

Fire

Flammable liquid. Use foam, dry powder, CO₂ or water spray. **Do not use water jet.**

Emergency contacts

Emergency services: 999

NHS non-emergency: 111

AMWS supervisor: Jason May

HSQE support: Sean Ashton (Onyx Operations)

9. Waste disposal

Hazardous waste (EWC 13 07 01*). Dispose via authorised waste-fuel contractor.

10. Risk rating

| Before controls | With controls |
| --- | --- |
| Moderate | Low |

Risk rating methodology: Severity × Likelihood = Risk (S × L = R). See POL_HSQE_21 — Risk Assessment Policy for the full scoring matrix.

11. Training requirements

Safe refuelling procedure

Spill response and use of spill kit

DSEAR awareness

Skin-care and dermatitis prevention

12. Monitoring and review

Weekly van check — fuel-cap condition, spill-kit stock

Monthly storage-area inspection

Annual COSHH review

Health surveillance for fuel-exposed roles (per APP_08)

13. References

POL_HSQE_08 — COSHH Policy

POL_HSQE_19 — Personal Protective Equipment (PPE) Policy

SOP 8.5 — Hazardous Substances (COSHH)

APP_09 — COSHH Assessments Log

HSE COSHH Regulations 2002 (SI 2002/2677, as amended)

HSE EH40/2005 Workplace Exposure Limits

CLP Regulation (EC) No 1272/2008 (as retained in UK law)

Assessment approved by: Aaron Mason, Director · 1 June 2026

This assessment must be reviewed annually or when circumstances change — whichever is sooner.

Uncontrolled when printed.