COSHH Assessment

Clarke Motor Oil SAE 10W/30

| Document Information |  |
| --- | --- |
| Document Reference | COSHH_25 |
| Issue Number | 2 |
| Issue Date | 25 September 2026 |
| Next Review | 1 June 2027 |
| Assessor | Sean Ashton (Onyx Operations) |
| Approved By | Aaron Mason, Director |

Key references ▸ Clarke-API-SF-10W-30-Motor-Oil-3050845.pdf — Manufacturer Safety Data Sheet ▸ Clarke-API-SF-10W-30-Motor-Oil-3050845.pdf — Manufacturer Safety Data Sheet ▸ Clarke International sheet for API SF 10W-30 Motor Oil, article 3050845, version 4 of 11/10/2024, filed 27/09/2026. It records the product as not classified, against H315 and H411 printed on Clarke's own label for the same article. The classification in this assessment is taken from the label. ▸ Van Packs portal — COSHH section — synchronised copy carried by every operative

1. Product Identification

| Product name | Clarke Motor Oil SAE 10W/30, API SF, 1 litre — article 3050845 |
| --- | --- |
| Part number | 3050845 |
| Supplier | Clarke International, Hemnall Street, Epping, Essex CM16 4LG |
| Pack | 1 litre |
| Specification | API SF. For 4-stroke petrol engines — pressure washers, generators, mowers and yard plant |
| Active ingredients / CAS | Severely hydrotreated / solvent-refined mineral base oil plus proprietary performance additives CAS: 64742-54-7 / 64742-65-0 (typical base stocks) |
| Physical form | Liquid |
| Appearance / odour | Amber viscous oil, characteristic mild odour |
| Use | Engine oil top-up and oil changes on the AMWS fleet and yard plant |
| Location | Jason May's COSHH cabinet, AMWS yard |

2. Hazard Classification

Overall classification: Classified under GB CLP. Skin Irrit. 2 and Aquatic Chronic 2 per the container label. Used engine oil is more hazardous still — it accumulates polycyclic aromatic hydrocarbons and is treated as a skin carcinogen, so every drain-down is the hazardous operation even though the classification below is for the fresh product. Signal word: Warning Hazard statements (H-codes) H315 Causes skin irritation. H411 Toxic to aquatic life with long lasting effects. Precautionary statements (P-codes) P264 Wash hands thoroughly after handling. P273 Avoid release to the environment. P280 Wear protective gloves and eye protection. P302+P352 IF ON SKIN: wash with plenty of water. P332+P313 If skin irritation occurs: get medical advice. P362 Take off contaminated clothing and wash it before reuse. P391 Collect spillage. P501 Dispose of contents and container to an authorised waste-oil contractor. GHS pictograms • Harmful • Dangerous for environment

3. Persons at Risk

4. Routes of Exposure

•  Inhalation: Negligible at ambient temperature. Oil mist only where sprayed or hot

•  Skin contact: The main route. Repeated or prolonged contact causes dermatitis. Used oil is a skin carcinogen — treat every drain-down as the hazardous operation

•  Eye contact: Transient irritation on splash

•  Ingestion: Unlikely in normal use

5. Workplace Exposure Limits

| Substance | 8-hr TWA | 15-min STEL |
| --- | --- | --- |
| Oil mist, mineral | 5 mg/m³ | 10 mg/m³ |

6. Frequency and Duration of Exposure

COSHH Regulation 6(2) requires the assessment to consider the amount, the level and the duration of exposure.

| Amount used | Times per day | Duration per use |
| --- | --- | --- |
| Medium — litres | 1–5 times per day | 6–30 minutes |

7. Control Measures — Hierarchy of Controls

Elimination / substitution

- Not practicable — the oil grade is specified by the engine manufacturer

Engineering controls

•  Drain pan and funnel for every oil change

•  Bunded storage in the COSHH cabinet

•  Spill kit within reach of the working position

Administrative controls

•  No hand-washing in oil or solvent — soap and water only

•  Skin checks form part of the health-surveillance conversation under SOP 8.7

•  Used oil never decanted into unmarked containers

Personal Protective Equipment (PPE)

8. Storage

| Temperature | Ambient |
| --- | --- |
| Location | Yard COSHH cabinet, bunded |
| Avoid | Strong oxidisers; contact with food and drink |
| Container | Original container, closed, clearly labelled |

9. Emergency Procedures

First aid

| Route | Action |
| --- | --- |
| Eyes | Rinse with clean water for several minutes, eyelids held open. Get medical advice if irritation persists. |
| Skin | Wash with soap and water. Do not use solvent or abrasive hand cleaner. Remove contaminated clothing and wash it before reuse. The product is a skin irritant — get medical advice if irritation continues after washing. |
| Inhalation | Not applicable in normal use. Move to fresh air if mist has been generated. |
| Ingestion | Rinse mouth. Do not induce vomiting. Seek medical advice. |

Spillage

Absorb with proprietary oil absorbent or sand. Protect the nearest drain first. The product is H411 — toxic to aquatic life with long lasting effects, so any release reaching the yard gully, a drain or a watercourse is a reportable environmental incident: report on FORM_ENV_01 and notify the Environment Agency incident hotline 0800 80 70 60. Dispose of the absorbent as hazardous waste. See SOP 9.3.

Fire

Combustible when heated. Foam, dry powder or CO₂. Do not use a water jet.

Emergency contacts

•  Emergency services: 999

•  NHS non-emergency: 111

•  AMWS supervisor: Jason May

•  HSQE support: Sean Ashton (Onyx Operations)

•  Environment Agency incident hotline: 0800 80 70 60 — if product reaches a drain, ditch or watercourse

10. Waste Disposal

Fresh oil: EWC 13 02 05*. Used oil: hazardous waste, EWC 13 02 05\* — authorised waste-oil contractor only, with a consignment note retained.

11. Risk Rating

Risk = Severity × Likelihood (S × L = R). See POL_HSQE_21 Risk Assessment policy for the full scoring matrix.

| Before controls — Moderate | With controls — Low (residual) |
| --- | --- |

12. Training Requirements

•  Safe handling and decanting

•  Dermatitis awareness and skin checks

•  Spill response — SOP 9.3

•  Used-oil handling and the carcinogen risk

13. Monitoring & Review

•  Weekly COSHH cabinet inspection by the Site Supervisor

•  Annual COSHH review, or sooner on change of product, supplier or task

•  Review immediately if the SDS obtained differs from the label classification recorded above

14. References

•  POL_HSQE_08 — COSHH Policy

•  POL_HSQE_19 — Personal Protective Equipment (PPE) Policy

•  SOP 8.5 — Hazardous Substances (COSHH)

•  SOP 8.7 — Health Surveillance

•  APP_09 — COSHH Assessments Log

•  HSE COSHH Regulations 2002 (SI 2002/2677, as amended)

•  HSE EH40/2005 Workplace Exposure Limits

•  CLP Regulation (EC) No 1272/2008 (as retained in UK law)

This assessment must be reviewed annually or sooner if circumstances change. For the full management-system context (SOPs, Appendices, policies) see the AMWS IMS portal.