COSHH Assessment

Grundo-Oil (pneumatic mole / hammer lubricant)

| Document Information |  |
| --- | --- |
| Document Reference | COSHH_26 |
| Issue Number | 1 |
| Issue Date | 8 September 2026 |
| Next Review | 1 June 2027 |
| Assessor | Sean Ashton (Onyx Operations) |
| Approved By | Aaron Mason, Director |

Key references ▸ GRUNDO-OIL-TT-Technologies-206115-USA-version.pdf — Manufacturer Safety Data Sheet ▸ TT Technologies sheet for GRUNDO-OIL, material 206115, version 2-0/USA of 22/04/2015, filed 27/09/2026. The manufacturer's own sheet, agreeing with the container label on the polyethylene glycol base and on the product being not classified. The EU sheet against GRU2002311 and GRU2002317 has been requested from TRACTO-TECHNIK for 31/10/2026. ▸ Van Packs portal — COSHH section — synchronised copy carried by every operative

1. Product Identification

| Product name | TRACTO GRUNDO-OIL — lubricant for GRUNDOMAT and GRUNDORAM, part GRU2002311 (5 l) / GRU2002317 (25 l) |
| --- | --- |
| Supplier | TRACTO-TECHNIK GmbH & Co. KG, Paul-Schmidt-Strasse 2, 57368 Lennestadt, Germany |
| Active ingredients / CAS | Polyethylene glycol, with less than 5% anionic surfactant. Water-soluble. Read from the container label photographed 24/09/2026. The manufacturer’s own safety data sheet gives the same base, polyethylene glycol of mean molecular weight 200 to 600 |
| Physical form | Liquid |
| Appearance / odour | Light amber oil, mild hydrocarbon odour |
| Use | In-line lubrication of GRUNDOMAT / GRUNDORAM pneumatic soil-displacement hammers used on moling and thrust-boring work |
| Location | Jason May's COSHH cabinet, AMWS yard |

2. Hazard Classification

Overall classification: To be confirmed from the SDS. Pneumatic tool oils of this type are typically either unclassified or classified for aspiration toxicity depending on viscosity. Signal word: None required — the manufacturer classifies the product as not a hazardous substance or mixture, and the container label carries no signal word and no pictogram Hazard statements (H-codes) — Not classified as hazardous in the supplied form Precautionary statements (P-codes) P262 Do not get in eyes, on skin or on clothing. P280 Wear protective gloves and eye protection. P273 Avoid release to the environment. P501 Dispose to an authorised waste-oil contractor.

3. Persons at Risk

4. Routes of Exposure

•  Inhalation: The significant route for this product. Pneumatic tools exhaust an oil mist at the tool and at the airline connections

•  Skin contact: Repeated contact causes dermatitis

•  Eye contact: Mist and pressurised release can carry oil to the eyes

•  Ingestion: Unlikely

5. Workplace Exposure Limits

| Substance | 8-hr TWA | 15-min STEL |
| --- | --- | --- |
| Oil mist, mineral | 5 mg/m³ | 10 mg/m³ |

6. Frequency and Duration of Exposure

COSHH Regulation 6(2) requires the assessment to consider the amount, the level and the duration of exposure.

| Amount used | Times per day | Duration per use |
| --- | --- | --- |
| Small — millilitres per fill | 1–5 times per day | 1–5 minutes to charge the lubricator; exposure continues while the tool runs |

7. Control Measures — Hierarchy of Controls

Elimination / substitution

- Not practicable — lubrication is required for tool life and safe operation

Engineering controls

•  In-line lubricator charged rather than oil poured into the air inlet

•  Work outdoors or in open excavation — the normal working environment disperses mist

•  Airline connections checked for leaks before the shift

Administrative controls

•  Do not stand in the tool exhaust stream

•  Charge the lubricator with the airline isolated and depressurised

•  Hand-arm vibration exposure is controlled separately under SOP 8.10

Personal Protective Equipment (PPE)

8. Storage

| Temperature | Ambient |
| --- | --- |
| Location | Yard COSHH cabinet, bunded |
| Avoid | Strong oxidisers; contamination with grit |
| Container | Original container, closed |

9. Emergency Procedures

First aid

| Route | Action |
| --- | --- |
| Eyes | Rinse with water for several minutes. Seek medical advice if a pressurised jet has contacted the eye. |
| Skin | Wash with soap and water. |
| Inhalation | Move to fresh air. Seek medical advice if breathing is uncomfortable. |
| Ingestion | Rinse mouth. Do not induce vomiting. Seek medical advice. |

Spillage

Absorb with oil absorbent. Protect the excavation and the nearest drain. Dispose as hazardous waste.

Fire

Combustible when heated. Foam, dry powder or CO₂.

Emergency contacts

•  Emergency services: 999

•  NHS non-emergency: 111

•  AMWS supervisor: Jason May

•  HSQE support: Sean Ashton (Onyx Operations)

10. Waste Disposal

Hazardous waste, EWC 13 02 05*. Authorised waste-oil contractor with a consignment note.

11. Risk Rating

Risk = Severity × Likelihood (S × L = R). See POL_HSQE_21 Risk Assessment policy for the full scoring matrix.

| Before controls — Moderate | With controls — Low (residual) |
| --- | --- |

12. Training Requirements

•  Pneumatic tool lubrication procedure

•  Airline safety — isolate and depressurise before disconnecting

•  Oil mist and dermatitis awareness

•  HAVS awareness — SOP 8.10

13. Monitoring & Review

•  Weekly COSHH cabinet inspection by the Site Supervisor

•  Annual COSHH review, or sooner on change of product, supplier or task

•  Review immediately if the SDS obtained differs from the profile assumed above

14. References

•  POL_HSQE_08 — COSHH Policy

•  POL_HSQE_19 — Personal Protective Equipment (PPE) Policy

•  SOP 8.5 — Hazardous Substances (COSHH)

•  SOP 8.7 — Health Surveillance

•  APP_09 — COSHH Assessments Log

•  HSE COSHH Regulations 2002 (SI 2002/2677, as amended)

•  HSE EH40/2005 Workplace Exposure Limits

•  CLP Regulation (EC) No 1272/2008 (as retained in UK law)

This assessment must be reviewed annually or sooner if circumstances change. For the full management-system context (SOPs, Appendices, policies) see the AMWS IMS portal.