COSHH Assessment

Hydraulic Oil ISO 32 Premium Mineral

| Document Information |  |
| --- | --- |
| Document Reference | COSHH_28 |
| Issue Number | 1 |
| Issue Date | 8 September 2026 |
| Next Review | 1 June 2027 |
| Assessor | Sean Ashton (Onyx Operations) |
| Approved By | Aaron Mason, Director |

Key references ▸ U412-Hydraulic-32-iss-10.pdf — Manufacturer Safety Data Sheet ▸ Records the product as not classified under GB CLP, matching both this assessment and the container label. Not held locally — save a copy to docs/coshh/sds/ so the assessment does not depend on an external URL. Owner: Leanne Mason, target 31/10/2026. ▸ Van Packs portal — COSHH section — synchronised copy carried by every operative

1. Product Identification

| Product name | ExoPro Hydraulic 32 Premium Mineral Hydraulic Fluid, 5 litre — part U41255L |
| --- | --- |
| Supplier | FPS, 2 Eskan Court, Campbell Park, Milton Keynes MK9 4AN |
| Active ingredients / CAS | Solvent-refined / hydrotreated mineral base oil with anti-wear additive package CAS: 64742-54-7 / 64742-65-0 (typical base stocks) |
| Physical form | Liquid |
| Appearance / odour | Clear to pale amber oil, mild hydrocarbon odour |
| Use | Hydraulic system top-up on the grab lorries, tipper bodies and yard plant |
| Location | Jason May's COSHH cabinet, AMWS yard |

2. Hazard Classification

Overall classification: Most ISO VG 32 hydraulic oils are not classified as hazardous under CLP in the supplied form. The hazard on AMWS work is not the oil in the drum — it is pressurised release. Signal word: None required for the fresh product Hazard statements (H-codes) — Not classified as hazardous in the supplied form Precautionary statements (P-codes) P262 Do not get in eyes, on skin or on clothing. P280 Wear protective gloves and eye protection. P273 Avoid release to the environment. P501 Dispose to an authorised waste-oil contractor.

3. Persons at Risk

4. Routes of Exposure

•  Inhalation: Negligible at ambient temperature; oil mist where a line fails under pressure

•  Skin contact: Repeated contact causes dermatitis

•  Eye contact: Irritation on splash

•  Injection: The serious hazard. A pinhole leak in a hydraulic line will inject oil through intact skin at working pressure. This is a surgical emergency and it does not look serious at first — see the first-aid note below

•  Ingestion: Unlikely

5. Workplace Exposure Limits

| Substance | 8-hr TWA | 15-min STEL |
| --- | --- | --- |
| Oil mist, mineral | 5 mg/m³ | 10 mg/m³ |

6. Frequency and Duration of Exposure

COSHH Regulation 6(2) requires the assessment to consider the amount, the level and the duration of exposure.

| Amount used | Times per day | Duration per use |
| --- | --- | --- |
| Medium — litres | 1–5 times per day | 6–30 minutes |

7. Control Measures — Hierarchy of Controls

Elimination / substitution

- Not practicable — grade is specified by the plant manufacturer

Engineering controls

•  Hydraulic hoses inspected as part of the daily walkaround check

•  Never search for a hydraulic leak with a hand — use a piece of cardboard held at arm's length

•  System depressurised before any hose is broken

•  Drip tray and funnel for top-up

Administrative controls

•  Hose condition and date recorded in the plant maintenance record

•  Leaks reported and the machine taken out of service, not run on

•  Spill kit within reach of the working position

Personal Protective Equipment (PPE)

8. Storage

| Temperature | Ambient |
| --- | --- |
| Location | Yard COSHH cabinet, bunded |
| Avoid | Strong oxidisers; contamination with grit or water |
| Container | Original container, closed and clearly labelled |

9. Emergency Procedures

First aid

| Route | Action |
| --- | --- |
| Eyes | Rinse with water for several minutes. |
| Skin | Wash with soap and water. |
| Injection injury | Call 999 and say the words 'high-pressure injection injury'. Do not wait to see whether it develops. The entry wound may be a pinprick and appear trivial; the injury underneath is not. Immobilise the limb, keep the casualty warm, and take the product container to hospital. |
| Inhalation | Move to fresh air if mist has been generated. |
| Ingestion | Rinse mouth. Do not induce vomiting. Seek medical advice. |

Spillage

Absorb with oil absorbent or sand. Protect the nearest drain first. Dispose as hazardous waste. See SOP 9.3.

Fire

Combustible when heated. Foam, dry powder or CO₂. A pressurised leak onto a hot surface is an ignition risk.

Emergency contacts

•  Emergency services: 999

•  NHS non-emergency: 111

•  AMWS supervisor: Jason May

•  HSQE support: Sean Ashton (Onyx Operations)

10. Waste Disposal

Hazardous waste, EWC 13 01 10*. Authorised waste-oil contractor with a consignment note.

11. Risk Rating

Risk = Severity × Likelihood (S × L = R). See POL_HSQE_21 Risk Assessment policy for the full scoring matrix.

| Before controls — Moderate | With controls — Low (residual) |
| --- | --- |

12. Training Requirements

•  Hydraulic system safety and the injection-injury hazard — the item on this assessment that most needs briefing

•  Daily walkaround hose inspection

•  Spill response — SOP 9.3

•  Dermatitis awareness

13. Monitoring & Review

•  Weekly COSHH cabinet inspection by the Site Supervisor

•  Annual COSHH review, or sooner on change of product, supplier or task

•  Review immediately if the SDS obtained differs from the profile assumed above

14. References

•  POL_HSQE_08 — COSHH Policy

•  POL_HSQE_19 — Personal Protective Equipment (PPE) Policy

•  SOP 8.5 — Hazardous Substances (COSHH)

•  SOP 8.7 — Health Surveillance

•  APP_09 — COSHH Assessments Log

•  HSE COSHH Regulations 2002 (SI 2002/2677, as amended)

•  HSE EH40/2005 Workplace Exposure Limits

•  CLP Regulation (EC) No 1272/2008 (as retained in UK law)

This assessment must be reviewed annually or sooner if circumstances change. For the full management-system context (SOPs, Appendices, policies) see the AMWS IMS portal.