COSHH Assessment

Silicone Lubricant — General Purpose (aerosol)

| Document Information |  |
| --- | --- |
| Document Reference | COSHH_29 |
| Issue Number | 1 |
| Issue Date | 25 September 2026 |
| Next Review | 1 June 2027 |
| Assessor | Sean Ashton (Onyx Operations) |
| Approved By | Aaron Mason, Director |

Key references ▸ 3975823.pdf — Manufacturer Safety Data Sheet ▸ Reached through a distributor copy because the CRC document server would not open. Records Danger, H222, H229, H315, H336, H411, matching the container label read on 24/09/2026. Not held locally — save a copy to docs/coshh/sds/. Owner: Leanne Mason, target 31/10/2026. ▸ Van Packs portal — COSHH section — synchronised copy carried by every operative

1. Product Identification

| Product name | Ambersil Silicone Lubricant — General Purpose (aerosol), 500 ml |
| --- | --- |
| Supplier | CRC Industries UK (Ambersil). WRAS Approved Product Licence No. 1811507 |
| Active ingredients / CAS | Polydimethylsiloxane in a hydrocarbon solvent carrier with liquefied petroleum gas propellant. The container label states: hydrocarbons, C7, n-alkanes, isoalkanes, cyclics CAS: 63148-62-9 (PDMS), 68476-86-8 (LPG propellant) |
| Physical form | Aerosol |
| Appearance / odour | Clear liquid spray, faint solvent odour |
| Use | General-purpose lubrication and release — hinges, seals, cable, sliding mechanisms, valve spindles |
| Location | Jason May's COSHH cabinet, AMWS yard |

2. Hazard Classification

Overall classification: Aerosol 1, with skin irritation, narcotic effects and chronic aquatic toxicity. Read from the container label photographed 24/09/2026. The supplier sheet records the same classification. The hazard is the hydrocarbon carrier and the propellant rather than the silicone itself. Signal word: Danger Hazard statements (H-codes) H222 Extremely flammable aerosol. H229 Pressurised container: may burst if heated. H315 Causes skin irritation. H336 May cause drowsiness or dizziness. H411 Toxic to aquatic life with long lasting effects. Precautionary statements (P-codes) P210 Keep away from heat, sparks, open flames and hot surfaces. No smoking. P211 Do not spray on an open flame or other ignition source. P251 Do not pierce or burn, even after use. P261 Avoid breathing spray. P271 Use only outdoors or in a well-ventilated area. P273 Avoid release to the environment. P280 Wear protective gloves. P302+P352 IF ON SKIN: wash with plenty of water. P332+P313 If skin irritation occurs: get medical advice. P391 Collect spillage. P410+P412 Protect from sunlight. Do not expose to temperatures exceeding 50 °C. P501 Dispose of contents and container to an authorised waste contractor. GHS pictograms • Flammable • Harmful • Dangerous for environment

3. Persons at Risk

4. Routes of Exposure

•  Inhalation: The main route. Spray mist and solvent vapour, particularly in a chamber or van

•  Skin contact: Mild defatting on repeated contact

•  Eye contact: Irritation from direct spray

•  Ingestion: Not a realistic route from an aerosol

5. Workplace Exposure Limits

| Substance | 8-hr TWA | 15-min STEL |
| --- | --- | --- |
| Hydrocarbon solvent carrier | Product-specific — confirm from the SDS | Product-specific |
| LPG propellant | 1000 ppm | 1250 ppm |

6. Frequency and Duration of Exposure

COSHH Regulation 6(2) requires the assessment to consider the amount, the level and the duration of exposure.

| Amount used | Times per day | Duration per use |
| --- | --- | --- |
| Small — millilitres | 1–5 times per day | 1–5 minutes |

7. Control Measures — Hierarchy of Controls

Elimination / substitution

- Use a brush-applied or wipe-on lubricant where the application allows — it removes the aerosol and the propellant entirely

Engineering controls

•  Use outdoors or in the open yard

•  Not to be used in a chamber, manhole or any confined space — the propellant is heavier than air and displaces it. See SOP 8.12

Administrative controls

•  Kept in the COSHH cabinet, away from heat

•  Never stored in a vehicle in warm weather — the container can burst above 50 °C

•  No hot work in the vicinity while spraying

•  Cans not pierced or crushed, even when empty

Personal Protective Equipment (PPE)

8. Storage

| Temperature | Below 50 °C — do not leave in a van in summer |
| --- | --- |
| Location | Yard COSHH cabinet, away from ignition sources |
| Avoid | Heat, direct sunlight, sparks, open flame |
| Container | Original pressurised container |

9. Emergency Procedures

First aid

| Route | Action |
| --- | --- |
| Eyes | Rinse cautiously with water for several minutes. Remove contact lenses if present. |
| Skin | Wash with soap and water. |
| Inhalation | Move to fresh air and keep at rest. Seek medical advice if drowsiness or dizziness persists. |
| Ingestion | Not a realistic route. Rinse mouth if contact occurs. |

Spillage

Ventilate. Eliminate ignition sources. Absorb any liquid with inert absorbent. Dispose as hazardous waste.

Fire

Extremely flammable aerosol. Do not spray towards an ignition source. Containers may burst if heated — cool with water spray from a protected position.

Emergency contacts

•  Emergency services: 999

•  NHS non-emergency: 111

•  AMWS supervisor: Jason May

•  HSQE support: Sean Ashton (Onyx Operations)

10. Waste Disposal

Hazardous waste — pressurised container, EWC 16 05 04*. Authorised contractor. Do not put in general waste, even when empty.

11. Risk Rating

Risk = Severity × Likelihood (S × L = R). See POL_HSQE_21 Risk Assessment policy for the full scoring matrix.

| Before controls — Moderate | With controls — Low (residual) |
| --- | --- |

12. Training Requirements

•  Aerosol safety and the confined-space prohibition

•  Storage temperature — the van issue

•  Waste route for pressurised containers

13. Monitoring & Review

•  Weekly COSHH cabinet inspection by the Site Supervisor

•  Annual COSHH review, or sooner on change of product, supplier or task

•  Review immediately if the SDS obtained differs from the profile assumed above

14. References

•  POL_HSQE_08 — COSHH Policy

•  POL_HSQE_19 — Personal Protective Equipment (PPE) Policy

•  SOP 8.5 — Hazardous Substances (COSHH)

•  SOP 8.7 — Health Surveillance

•  APP_09 — COSHH Assessments Log

•  HSE COSHH Regulations 2002 (SI 2002/2677, as amended)

•  HSE EH40/2005 Workplace Exposure Limits

•  CLP Regulation (EC) No 1272/2008 (as retained in UK law)

This assessment must be reviewed annually or sooner if circumstances change. For the full management-system context (SOPs, Appendices, policies) see the AMWS IMS portal.