COSHH Assessment

Engine and Parts Degreaser

| Document Information |  |
| --- | --- |
| Document Reference | COSHH_30 |
| Issue Number | 1 |
| Issue Date | 25 September 2026 |
| Next Review | 1 June 2027 |
| Assessor | Sean Ashton (Onyx Operations) |
| Approved By | Aaron Mason, Director |

Key references ▸ Halfords-Engine-and-Parts-Degreaser-621525.pdf — Manufacturer Safety Data Sheet ▸ Halfords-Engine-and-Parts-Degreaser-621525.pdf — Manufacturer Safety Data Sheet ▸ Halfords Ltd sheet for Engine & Parts Degreaser, product 621525, revision 1.02 of 18/05/2026, filed 27/09/2026. The GB supplier's current sheet, agreeing with the container label on Danger, H222, H229, H318. The classification in this assessment is corrected to it. ▸ Van Packs portal — COSHH section — synchronised copy carried by every operative

1. Product Identification

| Product name | Halfords Engine & Parts Degreaser (aerosol) — product code 621525 |
| --- | --- |
| Supplier | Halfords Ltd, Icknield Street Drive, Washford West, Redditch, Worcestershire B98 0DE |
| Active ingredients / CAS | Hydrocarbons, C11-C14, n-alkanes, isoalkanes, cyclics, less than 2% aromatics; isotridecanol, ethoxylated; amides, C8-18 and C18-unsaturated, N,N-bis(hydroxyethyl); n-butane propellant at 10%. Aliphatic hydrocarbons at 30% or more |
| Physical form | Aerosol |
| Appearance / odour | Clear liquid spray, solvent odour |
| Use | Degreasing engines, plant components and parts before maintenance or inspection |
| Location | Jason May's COSHH cabinet, AMWS yard |

2. Hazard Classification

Overall classification: Aerosol 1 and Serious Eye Damage 1. Read from the container label photographed 24/09/2026. The supplier sheet, revision 1.02 of 18/05/2026, records the same classification. Revision 1 of this assessment carried a generic hydrocarbon-degreaser profile instead, which is corrected below. Signal word: Danger Hazard statements (H-codes) H222 Extremely flammable aerosol. H229 Pressurised container: may burst if heated. H318 Causes serious eye damage. Precautionary statements (P-codes) P102 Keep out of reach of children. P210 Keep away from heat, hot surfaces, sparks, open flames and other ignition sources. No smoking. P211 Do not spray on an open flame or other ignition source. P251 Do not pierce or burn, even after use. P261 Avoid breathing vapour or spray. P271 Use only outdoors or in a well-ventilated area. P280 Wear eye protection and face protection. P305+P351+P338 IF IN EYES: rinse cautiously with water for several minutes. Remove contact lenses if present and easy to do. Continue rinsing. P310 Immediately call a POISON CENTRE or doctor. P410+P412 Protect from sunlight. Do not expose to temperatures exceeding 50 °C. P501 Dispose of contents and container in accordance with national regulations. GHS pictograms • Flammable • Corrosive

3. Persons at Risk

4. Routes of Exposure

•  Inhalation: Solvent vapour and spray mist. Narcotic effects at concentration — the risk rises sharply in an enclosed space

•  Skin contact: Defatting and irritation; repeated contact causes dermatitis. Degreasers strip skin oils by design

•  Eye contact: Irritation on splash or spray

•  Ingestion: May be fatal if swallowed and aspirated. Never decant into a drinks container

5. Workplace Exposure Limits

| Substance | 8-hr TWA | 15-min STEL |
| --- | --- | --- |
| Hydrocarbon solvent | Product-specific — confirm from the SDS | Product-specific |
| Oil mist, mineral | 5 mg/m³ | 10 mg/m³ |

6. Frequency and Duration of Exposure

COSHH Regulation 6(2) requires the assessment to consider the amount, the level and the duration of exposure.

| Amount used | Times per day | Duration per use |
| --- | --- | --- |
| Medium — litres | 1–5 times per day | 6–30 minutes |

7. Control Measures — Hierarchy of Controls

Elimination / substitution

- Ask the supplier for a water-based or citrus-based degreaser. For most yard degreasing this is a straight swap and it removes both the flammability and the eye-damage hazard

Engineering controls

•  Degrease outdoors or with the workshop door open — never in a closed unit

•  Parts washed over a tray so run-off is captured, not on the yard surface

•  Run-off must not reach the yard drain. The product is not classified as aquatic-toxic, but it is a hydrocarbon solvent and a discharge to a surface-water drain is still a pollution incident

Administrative controls

•  Never decant into an unlabelled or drinks container

•  No hot work or smoking in the area while degreasing

•  Spill kit within reach

•  Contaminated rags to the hazardous waste route, not general waste

Personal Protective Equipment (PPE)

8. Storage

| Temperature | Ambient; away from heat and ignition sources |
| --- | --- |
| Location | Yard COSHH cabinet, bunded, away from oxidisers |
| Avoid | Heat, sparks, open flame, strong oxidisers; food and drink containers |
| Container | Original labelled container, closed |

9. Emergency Procedures

First aid

| Route | Action |
| --- | --- |
| Eyes | Rinse cautiously with water for several minutes. Remove contact lenses if present. Seek medical advice if irritation persists. |
| Skin | Remove contaminated clothing. Wash with plenty of soap and water. Seek medical advice if irritation persists. |
| Inhalation | Move to fresh air and keep at rest. Seek medical advice if breathing is uncomfortable. |
| Ingestion | DO NOT induce vomiting — aspiration risk. Rinse mouth. Call 999. Take the container to hospital. |

Spillage

Protect the drain before anything else — this product is toxic to aquatic life. Absorb with inert absorbent, bag as hazardous waste. Eliminate ignition sources. Report to the Site Supervisor and record under SOP 8.1.

Fire

Flammable. Keep away from ignition sources. Foam, dry powder or CO₂. Do not use a water jet — it spreads the pool.

Emergency contacts

•  Emergency services: 999

•  NHS non-emergency: 111

•  AMWS supervisor: Jason May

•  HSQE support: Sean Ashton (Onyx Operations)

10. Waste Disposal

Hazardous waste, EWC 14 06 03* or 13 02 05* depending on the contaminant load. Authorised contractor with a consignment note. Contaminated absorbent and rags follow the same route.

11. Risk Rating

Risk = Severity × Likelihood (S × L = R). See POL_HSQE_21 Risk Assessment policy for the full scoring matrix.

| Before controls — High | With controls — Low (residual) |
| --- | --- |

12. Training Requirements

•  Solvent safety and ventilation

•  Drain protection and spill response — SOP 9.3

•  Dermatitis awareness and glove selection

•  Substitution review — water-based alternatives

13. Monitoring & Review

•  Weekly COSHH cabinet inspection by the Site Supervisor

•  Annual COSHH review, or sooner on change of product, supplier or task

•  Review immediately if the SDS obtained differs from the profile assumed above

14. References

•  POL_HSQE_08 — COSHH Policy

•  POL_HSQE_19 — Personal Protective Equipment (PPE) Policy

•  SOP 8.5 — Hazardous Substances (COSHH)

•  SOP 8.7 — Health Surveillance

•  APP_09 — COSHH Assessments Log

•  HSE COSHH Regulations 2002 (SI 2002/2677, as amended)

•  HSE EH40/2005 Workplace Exposure Limits

•  CLP Regulation (EC) No 1272/2008 (as retained in UK law)

This assessment must be reviewed annually or sooner if circumstances change. For the full management-system context (SOPs, Appendices, policies) see the AMWS IMS portal.