COSHH Assessment

EVO TOOL Temporary Linemarking Paint — Blue

| Document Information |  |
| --- | --- |
| Document Reference | COSHH_32 |
| Issue Number | 1 |
| Issue Date | 5 October 2026 |
| Next Review | 1 June 2027 |
| Assessor | Sean Ashton (Onyx Operations) |
| Approved By | Aaron Mason, Director |

Key references ▸ No safety data sheet is held for this colour. The EVO TOOL sheet covering the range has been requested from CID Group (Site Supplies & Janitorial) — owner Sean Ashton, target 31/10/2026. ▸ Van Packs portal — COSHH section — synchronised copy carried by every operative

1. Product Identification

| Product name | EVO TOOL Temporary Linemarking Paint — Blue |
| --- | --- |
| Supplier | CID Group — Site Supplies & Janitorial (EVO TOOL) |
| Product code | LM00110 |
| Active ingredients / CAS | Acrylic resin, pigments and fast-drying solvents in an aerosol propellant. Free from propane, butane, lead and cadmium (manufacturer's specification sheet). CAS: mixture — see safety data sheet |
| Physical form | Aerosol, 750 ml can (pack of 12) |
| Appearance / odour | Blue spray |
| Use | Temporary blue line-marking for site demarcation and services |

2. Hazard Classification

Overall classification: Flammable Aerosol Cat 1, Aspiration Hazard Cat 1, Skin Irritation Cat 2, Eye Irritation Cat 2, STOT SE 3 (narcotic effects), Aquatic Chronic Cat 2 Signal word: Danger The EVO TOOL range shares one formulation across its colours, so the same classification is applied to every colour AMWS holds. It is set at the most protective of the classifications previously recorded for these products, pending the manufacturer's sheet from CID Group. Hazard statements (H-codes) H222 Extremely flammable aerosol. H229 Pressurised container: may burst if heated. H304 May be fatal if swallowed and enters airways. H315 Causes skin irritation. H319 Causes serious eye irritation. H336 May cause drowsiness or dizziness. H411 Toxic to aquatic life with long lasting effects. Precautionary statements (P-codes) P210 Keep away from heat, hot surfaces, sparks, open flames and other ignition sources. No smoking. P211 Do not spray on an open flame or other ignition source. P251 Do not pierce or burn, even after use. P261 Avoid breathing dust/fume/gas/mist/vapours/spray. P280 Wear protective gloves/protective clothing/eye protection/face protection. P410+412 Protect from sunlight. Do not expose to temperatures exceeding 50 °C. P501 Dispose of contents/container to an authorised waste contractor in accordance with local regulations. GHS pictograms • Flammable • Harmful • Human Health • Dangerous for environment

3. Persons at Risk

•  Line-marking operatives

•  Site personnel

4. Routes of Exposure

•  Inhalation: Vapour/mist — may cause dizziness

•  Skin contact: Irritation on prolonged contact

•  Eye contact: Irritation

•  Ingestion: Aspiration hazard

5. Workplace Exposure Limits

No UK Workplace Exposure Limits (HSE EH40/2005) assigned to this product or its constituents.

6. Frequency and Duration of Exposure

COSHH Regulation 6(2) requires the assessment to consider the amount, the level and the duration of exposure.

| Amount used | Times per day | Duration per use |
| --- | --- | --- |
| Small — millilitres | 1–5 | 6–30 minutes |

7. Control Measures — Hierarchy of Controls

Elimination / substitution

Evaluated where practicable — see Section 12.

Engineering controls

•  Use in open air wherever possible

Administrative controls

•  No smoking or hot work nearby

Personal Protective Equipment (PPE)

•  Nitrile gloves

•  Safety glasses

8. Storage

| Temperature | Below 50 °C |
| --- | --- |
| Location | Cool ventilated store |
| Avoid | Heat, sparks, direct sunlight |
| Container | Original aerosol can |

9. Emergency Procedures

First aid

| Route | Action |
| --- | --- |
| Eyes | Rinse with water for 15 minutes. Seek medical advice. |
| Skin | Wash with soap and water. |
| Inhalation | Move to fresh air. |
| Ingestion | Do not induce vomiting. Seek immediate medical attention. |

Spillage

Eliminate ignition sources. Absorb with inert material. Dispose as hazardous waste.

Fire

Flammable aerosol. Use foam, CO₂ or dry powder.

Emergency contacts

•  Emergency services: 999

•  NHS non-emergency: 111

•  AMWS supervisor: Jason May

•  HSQE support: Sean Ashton (Onyx Operations)

10. Waste Disposal

Empty aerosols are hazardous (EWC 16 05 04*).

11. Risk Rating

Risk = Severity × Likelihood (S × L = R). See POL_HSQE_21 Risk Assessment policy for the full scoring matrix.

| Before controls — Moderate | With controls — Low (residual) |
| --- | --- |

12. Training Requirements

•  Aerosol handling

13. Monitoring & Review

•  Weekly van-pack inspection

•  Annual COSHH review

14. References

•  POL_HSQE_08 — COSHH Policy

•  POL_HSQE_19 — Personal Protective Equipment (PPE) Policy

•  SOP 8.5 — Hazardous Substances (COSHH)

•  APP_09 — COSHH Assessments Log

•  HSE COSHH Regulations 2002 (SI 2002/2677, as amended)

•  HSE EH40/2005 Workplace Exposure Limits

•  CLP Regulation (EC) No 1272/2008 (as retained in UK law)

This assessment must be reviewed annually or sooner if circumstances change. For the full management-system context (SOPs, Appendices, policies) see the AMWS IMS portal.